Project Name: Enterprise RFID Smart Attendance System

Client: Digontom Technologies

Developed By: Gautam Pal

Date: April 2026

System Status: Production-Ready & Live

## 1. System Overview

The Smart RFID Attendance System is an IoT-based solution designed to log student/employee attendance in real-time. The hardware directly communicates with a cloud-hosted dashboard via Wi-Fi, eliminating the need for manual data entry. It provides instant audio-visual feedback to users and ensures high-security data logging.

## 2. Hardware Architecture

The physical device is built using the following core components:

- Microcontroller: ESP32 Development Board (Wi-Fi Enabled)

- Scanner: RFID RC522 Module (13.56MHz)

- Feedback System: Active Buzzer, Green LED (Success), Red LED (Error/Offline)

## 3. Key Features & Security

- Real-Time Cloud Sync: Data is instantly uploaded to the Next.js API hosted on Vercel.

- Duplicate Tap Prevention: The system ignores multiple taps from the same card within a 5-second window to prevent duplicate logging.

- Audio-Visual Feedback:

  - Success (Registered Card): 1 short beep + Green LED.

  - Failure (Unknown Card): 2 short beeps + Red LED.

- Offline Protection System: If the internet connection drops, the system triggers a continuous long beep and lights up the Red LED to alert the administration.

## 4. Setup & Configuration Guide (For Admin)

This device features a "Plug & Play" smart configuration. It does not have hardcoded Wi-Fi credentials.

Step-by-Step Initial Setup:

- Power On: Connect the device to a power source.

- Access Hotspot: Using a smartphone or laptop, search for available Wi-Fi networks and connect to "Smart_Attendance_System".

- Configure Network: A login page will automatically open. Select your local office/school Wi-Fi network and enter the password.

- Ready Mode: The device will restart, connect to the internet, and be ready to read cards. (Note: To change the Wi-Fi network in the future, simply restart the device and repeat this process).

## 5. Software & Dashboard

- Platform: Custom Next.js Web Application

- Hosting: Vercel Cloud Network

- Dashboard Features: * Live student/employee count.

  - Real-time table updating with Name, Roll No, Card UID, Date, and Time.

  - Premium Enterprise UI (Dark Theme).

- Live Access URL: https://attendance-app-oeht.vercel.app